ABSTRACT

This thesis presents the design, implementation, and evaluation of a cryptographic processor, the CryptoCPU, which exclusively executes encrypted programs by employing the AES-128 algorithm at the instruction fetch stage of a six-stage pipeline (Fetch → Decrypt → Decode → Execute → Memory → Write-Back). The processor targets the Artix-7 FPGA and was developed using the Vitis Unified and Vivado Design Suite toolchains.

Application code is written in a MIPS-like assembly language and processed through MARS, producing two outputs: .data.hex (unencrypted data section) and .text.hex (instructions). The .text section is encrypted with AES-128 in 16-byte blocks, while the .data section remains unmodified. Both are then merged into a single demo.hex file, which is loaded into the CryptoCPU’s memory. During execution, instructions are dynamically decrypted on-the-fly in the Decrypt stage, ensuring that plaintext instructions never appear in memory or on the FPGA buses.

To validate system functionality, a C++ testbench (test.cpp) was developed to supply the processor with the encrypted program and verify correct instruction loading and execution. The testbench was integrated into all stages of the Vitis flow (C-simulation, C-synthesis, RTL Co-simulation, IP packaging, and implementation), providing consistent evaluation of the CryptoCPU.

This thesis documents:

(a) the theoretical background (FPGAs, pipelines and hazards, instruction/data caches, AES),
(b) the architecture and specifications of the CryptoCPU (MIPS-like ISA with AES instructions, memory organization, cache policy, hazards),
(c) the complete toolflow MARS → AES encryption → testbench → Vitis (C-sim, C-synth, RTL Co-sim) → IP packaging → Vivado block design,
(d) simulation and implementation results (resource utilization, timing analysis),
(e) the integration into the Vivado block design (clocking wizard, reset logic, CryptoCPU IP, I/O pins), and
(f) testing on a real Artix-7 FPGA board, where correct functionality was confirmed by the activation of the led_done signal.

The results demonstrate that the design is fully synthesizable and implementable on FPGA, meeting timing constraints while using resources efficiently. Furthermore, the methodology shows that designing processors with encrypted instruction sets is feasible, secure, and practically deployable. Finally, possible extensions are discussed, including different AES modes, improved branch prediction, larger caches, and countermeasures against side-channel attacks.

Keywords: AES-128, FPGA Artix-7, MARS, MIPS-like ISA, testbench, Vitis Unified, Vivado, block design, on-the-fly decryption, pipeline.

Table of Contents

ABSTRACT	i

ABSTRACT	iii

CHAPTER 1 Introduction	2

(Subchapter 1.1) Objectives and Motivations	2

(Section 1.1.A) Motivations for Designing Cryptographic Processors	2

(Section 1.1.B) CRYPTOCPU Design Objectives	3

(Subsection 1.2) Contribution and Structure of the Thesis	5

(Section 1.2.A) Contribution of the Thesis	5

(Section 1.2.B) Chapter Structure	7

CHAPTER 2 Literature Review	9

(Subsection 2.1) Theoretical Framework	9

(Section 2.1.A) FPGA: characteristics and advantages	9

(Section 2.1.B) Pipeline in processors – Hazard and techniques	10

(Section 2.1.C) AES Algorithm and Cryptographic Applications	12

(Subsection 2.2) Related Research	14

(Section 2.2.A) Cryptographic processors in FPGA	14

Comparison Table: CRYPTOCPU vs Traditional Architectures	16

(Section 2.2.B) Security techniques with on-the-fly decryption	17

CHAPTER 3 Processor Design	19

(Subchapter 3.1) Architecture and Pipeline	19

(Section 3.1.A) ISA and Special Instructions	19

(Section 3.1.B) i-cache and d-cache implementation	20

(Section 3.1.C) AES integration in the Decrypt stage	22

(Section 3.1.D) Hazards handling (Data & Control Hazards)	24

(Subchapter 3.2) Tool Flow and Input Files	25

(Section 3.2.A) Assembly code and MARS	25

(Section 3.1.B) Encryption and demo.hex generation	27

CHAPTER 4 Vitis & Vivado	29

(Subsection 4.1) Vitis	29

(Section 4.1.A) C Simulation	29

(Section 4.2.B) C Synthesis	31

(Section 4.2.C) RTL Co-Simulation	31

(Section 4.2.D) IP Packaging	32

(Section 4.2.E) Implementation	34

(Subsection 4.2) Vivado	35

(Section 4.2.A) Block Design	35

(Section 4.2.B) Pin Assignment	37

(Section 4.2.C) Implementation Reports	38

(Section 4.2.D) Execution on a real board	39

CHAPTER 5 Conclusions	40

BIBLIOGRAPHY	42

CHAPTER 1 Introduction

(Subchapter 1.1) Objectives and Motivations

(Section 1.1.A) Motivations for Designing Cryptographic Processors

The need for enhanced security in digital systems has made the execution of encrypted instructions a critical issue, especially in applications where code integrity and confidentiality are as important as computational performance. In this context, cryptographic processors are an emerging class of architectures that aim to execute protected programs without revealing their internal contents.

Implementing security at the hardware level, and specifically within the pipeline, offers advantages that cannot be achieved with software techniques alone. The reasons why integrating cryptography into the processor itself is of strategic importance include:

- Prevention of code theft: When instructions remain encrypted until execution, the possibility of reading by malicious monitoring devices is eliminated.

- Reducing the attack surface: Blocking plaintext code from memory and buses reduces the points at which an attacker can intervene.

- Protection against reverse engineering: The use of dynamic (on-the-fly) decryption within the pipeline drastically reduces the possibility of successful reverse engineering through side-channel or timing attacks.

- Security by design: The processor operates on the assumption that nothing is trusted until it is decrypted within the controlled pipeline.

The conventional approach—storing encrypted instructions but decrypting them before the pipeline—creates a critical "window of vulnerability" where instructions are in plaintext form. The solution proposed in this paper through CryptoCPU fills this gap by introducing a new execution model:

- Instructions are stored exclusively in encrypted form and are only decrypted at the Decrypt stage of the pipeline.

- The AES-128 algorithm is integrated as an integral part of the architecture and does not function as an independent peripheral unit.

- No instruction is ever available in plaintext outside the strictly controlled environment of the pipeline.

This approach allows the creation of self-defending processors, which are suitable for critical applications such as:

- Secure embedded systems (e.g., in IoT or medical devices),

- Financial and military infrastructures, where command disclosure can lead to serious breaches,

- Cloud and virtualization systems, where hiding user software from the provider is desirable.

In summary, the design of cryptographic processors such as CryptoCPU embeds security principles into the hardware itself, creating a new level of protection that goes beyond traditional solutions. This work demonstrates that such architectures are not only necessary but also technically feasible with acceptable performance, even on resource-constrained platforms.

(Section 1.1.B) CRYPTOCPU Design Goals

This work aims to design and implement a secure processor that exclusively executes encrypted programs. The design objectives of CryptoCPU were defined based on the need for secure software execution on reconfigurable platforms such as FPGAs, without compromising performance or compatibility. In detail, the main objectives include:

1. Integration of AES-128 into the processor pipeline

The main objective is the full integration of the AES-128 algorithm into the internal pipeline of the CryptoCPU, with the creation of a new decryption stage (Decrypt). Specifically:

- The Decrypt stage is placed between the Fetch and Decode stages, processing each 16-byte block of instructions retrieved from memory.

- Decryption is performed on-the-fly, ensuring that commands remain encrypted across the entire memory and bus range.

- Decrypt does not function as an external unit but as an integral part of the pipeline, maintaining the consistency of the execution flow.

2. Compatibility with MIPS-like assembly

To facilitate user adoption, the architecture was designed to be compatible with a MIPS-like Instruction Set Architecture (ISA):

- Application development is performed using MARS (MIPS Assembler and Runtime Simulator), a familiar and easy-to-use environment.

- The choice of MIPS-like ISA allows for rapid code development while maintaining clarity and simplicity of instructions.

- The assembly syntax remains familiar, offering educational value and the ability to immediately utilize existing examples.

3. Separation of data and instructions

An important feature is the separation between instructions (.text) and data (.data):

- Only the .text section is subject to AES encryption, as it concerns the executable code.

- The .data section remains in plaintext, allowing normal access to variables and values during execution.

- This architecture of maintaining separation allows for simplicity in memory handling without compromising the security of the instruction flow.

4. Implementability on FPGA

One of the main goals is to prove that CryptoCPU can be fully implemented on a real FPGA platform. More specifically:

- The design targets the Xilinx Artix-7 board, which is often used in educational and research environments.

- The entire development process is carried out using Vitis Unified tools for high-level synthesis (HLS) and Vivado Design Suite for synthesis, placement, and hardware implementation.

- The successful implementation of CryptoCPU on FPGA proves that this approach is not theoretical but entirely practical and applicable.

5. Key validity check

To enhance security, a cryptographic key validity check mechanism is introduced:

- The AES key is loaded from a predefined memory block when the processor starts up.

- Before executing any encrypted code, the key is checked for authenticity.

- If a corrupted or incorrect key is detected, execution is interrupted, preventing tampering or violation of the application.

6. Proof of practical efficiency and correct operation

Finally, a significant part of the work focuses on the experimental verification and measurement of the CryptoCPU's operation:

- The test.cpp file is used as a testbench and loads the encrypted program (demo.hex) into the processor's memory.

- After decryption, a signature validity check is performed to verify that the value of memory[156] is equal to 0x15.

- The simulations (C Simulation) and Vitis logs confirm that the Decrypt stage runs without errors, and the overall operation is completely predictable and accurate.

- This process demonstrates the efficiency of the architecture and paves the way for integration into more complex systems.

In summary, the design of the CryptoCPU was based on specific, measurable, and achievable goals. The processor architecture combines security, compatibility, functionality, and scalability, making it suitable for future development of secure embedded systems on FPGAs.

(Subsection 1.2) Contribution and Structure of the Thesis

(Section 1.2.A) Contribution of the Thesis

This thesis attempts to fill an important gap in the field of software security and the execution of encrypted programs on reconfigurable FPGA platforms. Its contribution lies not only in the architectural innovation of CryptoCPU, but also in its complete implementation, verification, and practical application in real hardware. More specifically:

1. Architectural design of a processor with an eight-stage pipeline and integrated AES-128

The most fundamental contribution of the work is the design of a processor with a six-stage pipeline (Fetch → Decrypt → Decode → Execute → Memory → Write-Back), which incorporates the AES-128 algorithm as an integral part of its architecture.

- The placement of the Decrypt stage within the pipeline enables on-the-fly decryption of instructions without the need for an external decryption unit.

- The result is a secure processor in which no instruction ever appears in plaintext form in memory or on the buses, ensuring absolute confidentiality.

2. Integrated toolchain for secure conversion of assembly to executable .hex

The project implements a complete toolchain that connects the software level (assembly) with the hardware implementation (FPGA):

- The code is first written in MIPS-like assembly using the MARS tool, which exports two files, .text.hex and .data.hex.

- The .text.hex file goes through an encryption program (encryptor.cpp), which applies AES-128 in 16-byte blocks.

- The two pieces are merged into the final demo.hex file, which is loaded into the CryptoCPU's memory.

- The process is reproducible, autonomous, and fully documented, making it suitable for reuse or extension.

3. Execution of exclusively encrypted programs with key control

Unlike other architectures where decryption is optional or external, CryptoCPU requires encrypted code to execute anything.

- The start of execution includes an AES key validity check, which prevents accidental or malicious execution of corrupted commands.

- This design implements security by design, introducing a self-defense mechanism for the architecture before we even get to issues of operation or use.

4. Integration into FPGA with full verification and implementation chain

The work was not limited to a theoretical approach but was successfully completed on a real FPGA platform. The process included the following steps:

- C Simulation: testing the basic operation of CryptoCPU and decryption at the C++ level.

- C Synthesis: translation of the code to RTL level and production of hardware description.

- RTL Co-Simulation: interfacing the C++ testbench with RTL units to confirm behavior.

- IP Packaging: conversion of the CryptoCPU unit into a reusable IP block.

- Vivado Block Design & Implementation: placement of IP in actual system and assignment of pins, clock, reset, etc.

This complete flow demonstrates that the design is mature for integration into larger SoC systems or other applications.

5. Successful operation on a real FPGA board (Xilinx Artix-7)

The final and most tangible proof of CryptoCPU's success is its operation on the actual Xilinx Artix-7 board.

- After synthesizing the bitstream and transferring it to the target, the led_done signal was activated, confirming the correct execution of the encrypted program.

- The results matched the simulations, proving that the system remains reliable, stable, and consistent even in low-level implementation.
Overall contribution assessment

This work offers a holistic approach to the design of secure processors, from the AES code and algorithm to the hardware implementation. The creation of a MIPS-like CryptoCPU with exclusive execution of encrypted instructions, fully functional in FPGA, is a tangible step towards combining security and performance in modern reconfigurable architectures.

(Section 1.2.B) Chapter Structure

This thesis is structured in five main chapters, which are organized in a way that gradually guides the reader from the theoretical basis and motivations to the design, implementation, and evaluation of the CryptoCPU. Each chapter is designed to be self-contained, technically coherent, and at the same time interconnected with the others, offering a comprehensive picture of the project.

CHAPTER 1: INTRODUCTION, OBJECTIVES, AND MOTIVATIONS

The first chapter lays the foundations for the work:

•	It introduces the context of reconfigurable systems and the need for security in FPGAs.

•	It presents the motivations and design objectives of CryptoCPU.

•	Includes the contribution of the work, as well as a brief description of its structure.

CHAPTER 2: LITERATURE REVIEW

The second chapter presents the theoretical background:

•	The characteristics and advantages of FPGAs.

•	The concept of pipeline in processors, along with the related hazards and ways to deal with them.

•	The AES algorithm and its use in hardware.

•	Relevant research activity in cryptographic processors and on-the-fly decryption techniques.

CHAPTER 3: PROCESSOR DESIGN

The third chapter presents the core of the CryptoCPU:

•	The architecture, ISA, pipeline, and cache memories are described.

•	It analyzes the special Decrypt stage and the integration of AES.

•	The handling of data and control hazards is presented.

•	A detailed description of the tool flow is provided, from assembly writing in MARS to the production of demo.hex through encryption.

•	The operation of encryptor.cpp and test.cpp is examined, as well as the offline verification process.

CHAPTER 4: VITIS AND VIVADO

The fourth chapter refers to the practical implementation:

•	The flow in Vitis HLS is presented: from C Simulation to IP Packaging and the export process.

•	Describes block design and integration in Vivado.

•	Emphasis is placed on the implementation of CryptoCPU on a physical board (FPGA), as well as on implementation reports that document its functionality and performance.

CHAPTER 5: CONCLUSIONS

The final chapter summarizes:

•	The main results of the work.

•	The advantages of the CryptoCPU architecture, as well as the limitations observed.

•	Proposals for future expansion, such as support for new AES modes, the addition of branch prediction, and the implementation of countermeasures against side-channel attacks.

CHAPTER 2 Literature Review

(Subchapter 2.1) Theoretical Framework

(Section 2.1.A) FPGA: characteristics and advantages

Field Programmable Gate Arrays (FPGAs) are a key pillar of reprogrammable hardware, providing the ability to configure the circuit even after it has been manufactured. This flexibility makes them ideal for a wide range of applications, from embedded systems and robotics to cryptography and security.

FPGA architecture

An FPGA consists of:

- Logic Cells: A combination of LUTs and Flip-Flops for implementing logic functions and states.

- Routing: Allows arbitrary connections between cells.

- Configuration Memory: Contains the bitstream that determines the behavior of the circuit.

- Embedded Units: E.g., DSPs, Block RAMs, and ARM cores in modern FPGAs (such as Zynq SoCs).

Advantages over CPU/GPU/ASIC

| Feature | FPGA | CPU/GPU | ASIC |
| --- | --- | --- | --- |
| Reprogrammable | Yes | Yes (via software) | No |
| Parallel execution | Full | Limited | Full |
| Performance per Watt | Good | High (GPU) | Excellent |
| Development time | Average | Faster | Very long |
| Cost for small series | Low | Very low | Very high |
| Adaptability | High | Limited | None |

The flexibility and parallelism offered by FPGAs make them suitable for:

- Signal and image processing

- Machine learning accelerators

- Cryptographic modules

- Embedded real-time systems

FPGAs for Secure Implementations – Correlation with CryptoCPU

FPGAs provide critical advantages for implementing secure processors such as CryptoCPU:

- Hardware-based security: Algorithms (e.g., AES) are implemented directly in the hardware, offering speed and isolation.

- Side-channel protection: Ability to implement masking, randomization, and isolation techniques.

- Ease of upgrade: Ability to support new algorithms without changing hardware.

Challenges and Limitations

- Speed: Lag behind ASIC in frequency.

- Power consumption: Higher at full load.

- Development requirements: Requires knowledge of HDL (e.g., VHDL/Verilog).

- Bitstream security: Configuration protection is critical.

Conclusion

FPGAs bridge the gap between general-purpose processing and specialized hardware, offering a unique platform for architectures such as CryptoCPU, where security, speed, and adaptability are essential features.

(Section 2.1.B) Pipelines in processors – Hazards and techniques

The technique of instruction pipelining is a cornerstone of modern processor design, as it aims to increase performance through the simultaneous execution of multiple instructions. The pipeline separates the instruction execution process into distinct stages, allowing different instructions to be at different stages of processing at the same time.

The classic implementation includes five basic stages:

- Fetch (IF): Retrieving an instruction from memory.

- Decode (ID): Analysis of the instruction and reading of registers.

- Execute (EX): Execution of arithmetic/logical operations.

- Memory (MEM): Reading or writing from/to memory.

- Write Back (WB): Writing the result back to the registers.

CryptoCPU introduces a sixth stage – Decrypt, which is inserted between Fetch and Decode, so that instructions are decrypted before decoding. The integration of the Decrypt stage imposes new synchronization requirements and creates additional latency that must be addressed appropriately through pipeline optimization techniques.

Types of hazards

When executing multiple instructions in a pipeline, conflicts called hazards occur, which can be divided into three basic categories:

- Data Hazards: When an instruction depends on the result of a previous one that has not yet been completed.

Example:

add $t0, $t1, $t2

sub $t3, $t0, $t4   # $t0 is not yet available

If not addressed, the sub instruction will use an incorrect value.

- Control Hazards: These mainly arise from branch instructions (e.g., beq, j), as the pipeline does not know in advance which instruction to fetch next.

- Structural Hazards: When two or more instructions attempt to use the same system resource at the same time (e.g., shared memory or shared ALU).

Techniques for dealing with hazards

To address the above problems, specific pipeline optimization techniques have been developed:

- Data Forwarding (or Bypassing): The results from the Execute or Memory stages are forwarded directly to the next instructions that need them, without waiting for them to be written to the register file first.

- Pipeline Stalls (or NOP insertion): When forwarding is not possible, artificial delays are inserted into the pipeline to give the instructions time to complete.

- Branch Prediction: A branch prediction mechanism (static or dynamic) that attempts to guess what the next instruction will be so that the pipeline does not stall. In this paper, simple stalls are used, but the future integration of a dynamic branch predictor is suggested.

- Separate Caches (i-cache and d-cache): The use of separate memory for instructions and data prevents structural hazards, as it allows parallel access to different types of information.

Specifically for CryptoCPU

The integration of the Decrypt stage requires special attention to the handling of data and control hazards, as AES decrypts each block of instructions dynamically. This adds latency to the pipeline, which is minimized by using techniques such as #pragma HLS pipeline and #pragma HLS dataflow during HLS compilation (more in Chapter 4).

The CryptoCPU pipeline stage is as follows:

Fetch → Decrypt → Decode → Execute → Memory → Write-Back

The existence of Decrypt requires new forward paths and hazard handling mechanisms

hazards in order to maintain performance and correct execution.

(Section 2.1.C) AES algorithm and cryptographic applications

The Advanced Encryption Standard (AES) is the dominant symmetric encryption algorithm worldwide, as it was adopted as an official standard by the NIST (National Institute of Standards and Technology) in 2001. AES was selected through a competitive process, replacing the older DES, mainly due to its increased security and better performance.

Key features of AES-128

This paper uses AES-128, which is the version of the algorithm that uses a 128-bit key and encrypts 128-bit data blocks. The algorithm is based on iterative rounds, during which the initial block is gradually transformed until the final encrypted result is obtained.

The process consists of the following steps (per round):

- SubBytes: Non-linear replacement of each byte of the state through a substitution table (S-box). It offers non-linearity and contributes to security against linear and differential attacks.

- ShiftRows: Circular shift of the rows of the state. Increases diffusion and ensures that bytes interact in subsequent stages.

- MixColumns: Application of a linear transformation (multiplication in the GF(2⁸) field) per column. Contributes to further diffusion between bits.

- AddRoundKey: XOR of the state with the corresponding round key from the initial key expansion (Key Expansion).

AES-128 performs 10 rounds, and a total of 11 round keys are required. The round keys are generated from the initial secret key through the Key Expansion process, which includes operations such as RotWord, SubWord, Rcon, and XOR with previous rounds.

Modes of Operation

AES as a block cipher is not used on its own. Instead, it is implemented with a mode of operation so that it can process messages larger than 128 bits and increase security. The basic modes are:

- ECB (Electronic Codebook): The simplest form, where each block is encrypted independently. Although fast and easy to implement, it suffers from security issues, as the same plaintext blocks produce the same ciphertexts, leaving patterns visible.

- CBC (Cipher Block Chaining): Each input block is XORed with the previous ciphertext before encryption. This significantly increases security, but requires a dependency chain.

- CTR (Counter mode): Uses an incrementing counter and encrypts the counter, then XORs it with the plaintext. Provides high parallel execution.

- GCM (Galois Counter Mode): Offers both encryption and authentication, making it suitable for high-security applications.

ECB mode selection in CryptoCPU

In the context of this work and CryptoCPU, ECB mode was selected for decrypting commands. Although it lags behind other modes in terms of security, it offers:

- Simplicity of implementation in hardware: Each block is processed independently, without the need for a previous state or external memory.

- Lower implementation cost and better latency.

- Suitable application for instruction flow protection, where content changes frequently and patterns are difficult to predict.

Future versions of CryptoCPU may incorporate more secure modes, such as CTR or GCM, with parallel reinforcement of the architecture.

AES applications in FPGA

Implementing AES in FPGA offers significant advantages, especially in embedded and real-time applications. Some key areas of application include:

- VPNs and IPSec/SSL protocols: Encryption/decryption of network packets at gateways or routers.

- Data storage (Disk Encryption): Protection of sensitive files on SSD, Flash, and SD cards.

- Secure microcontrollers (Secure Embedded Systems): Platforms for bank cards, medical implants, smart IoT devices.

- Hardware root of trust: Integration of AES in FPGA as a key component of secure boot.

Speed, parallelism, energy efficiency, and low latency make FPGAs ideal for implementing AES, offering real-time encryption without the need for external processors.

(Subsection 2.2) Related Research

(Section 2.2.A) Cryptographic processors in FPGAs

The need to accelerate cryptographic operations in embedded and real-time systems has led to the rapid development of cryptographic processors and accelerators in FPGAs. Field Programmable Gate Arrays offer the ideal compromise between performance, flexibility, and cost, making them the preferred platform for implementing algorithms such as AES.

Indicative research efforts

- Tillich & Herbst (2003):
One of the first high-performance implementations of AES on FPGAs, aiming to achieve throughput exceeding 5 Gbps on Xilinx Virtex systems. This study highlighted the potential of FPGAs in mass data processing, especially for high-security environments such as military communications.

- Gaj & Chodowiec (2009):
They conducted an extensive comparison of FPGA and ASIC in terms of AES implementation. They showed that although ASICs are superior in speed and power consumption, FPGAs are superior in flexibility, development cost, and rapid prototyping capability. They also emphasized that in applications requiring continuous updating of algorithms or security parameters, FPGAs are the only way to go.

- Modern implementations (2020+):
The trend is shifting towards integrating AES as a co-processor alongside the main processor. The AES co-processor receives data from the CPU, encrypts/decrypts it, and returns the results. Communication is usually via shared memory or dedicated buses.

Limitations of traditional approaches

Although co-processors are efficient in terms of performance, they present certain structural and architectural problems:

- Increased data management complexity: The CPU must transfer data to the AES accelerator, wait for the response, and synchronize execution. This places a burden on the software and controller.

- Data security in the bus: When communication between the CPU and co-processor takes place via shared buses or shared memory, there is an increased risk of interception. Data may temporarily appear in plaintext form, opening up the possibility of side-channel attacks, especially in physically accessible systems.

- Inefficiency in real-time applications: The constant exchange between the CPU and AES co-processor introduces latency, which is critical in applications that require fast response times, such as security systems or multimedia streaming.

The CryptoCPU approach

CryptoCPU addresses these issues through a completely different architectural philosophy: it does not consider encryption as an external or supplementary function but integrates it intrinsically into the pipeline core. Specifically:

- The AES-128 algorithm is integrated directly into the Decrypt stage, which is placed between Fetch and Decode.

- The instruction flow always remains encrypted in memory and on the buses. Decryption is performed on-the-fly, just before decoding.

- No communication with external units is required, resulting in zero risk of data leakage during execution.

- This approach significantly reduces buffer memory, wait cycles, and vulnerability to attacks.

Conclusion

The research literature has focused mainly on accelerating AES or adding co-processors, often overlooking the issue of holistic protection of the instruction flow. CryptoCPU proposes an organic architecture in which decryption is not "something added on," but is a core element of the processor's operation.

This makes it suitable for:

- Integration into secure embedded systems (smart cards, IoT controllers, digitally certified modules).

- Development of advanced compiler toolchains that compile code directly into encrypted instructions.

- Future architectures with a vision of running exclusively encrypted software, drastically limiting reverse engineering.

Comparison Table: CRYPTOCPU vs. Traditional Architectures

| Criterion | CryptoCPU (with AES in the pipeline) | Traditional processor + AES co-processor |
| --- | --- | --- |
| AES placement | Integrated into the pipeline (Decrypt stage) | External unit |
| Instruction flow security | Commands always encrypted | Plaintext commands |
| Resistance to side-channel attacks | High – less data exposure in channels | Moderate – plaintext transfer to AES |
| CPU ↔ AES communication time | Zero (within pipeline) | High – requires handshaking and buffer |
| Software layer simplicity | No calls to external unit required | Requires data transfer instructions |
| Data reliability | Fully controlled flow | Possible inconsistency due to delays |
| Supports only encrypted programs | Yes (strict) | No – plaintext execution possible |
| Extensibility (e.g., AES modes) | Limited without pipeline redesign | Flexible (with new co-processor module) |
| Impact on pipeline | Small (Decrypt is a fixed stage) | None – but introduces external delay |
| Power consumption | Slightly higher due to integrated AES | Varies depending on access frequency |
| Overall complexity | Moderate – pipeline-aware design | High – requires interfaces and synchronization |
| Suitable for real-time applications | Yes | Not always – depends on latency |

(Section 2.2.B) Security techniques with on-the-fly decryption

Protecting executable code is a critical requirement in applications that rely on the confidentiality and integrity of programs. Traditional protection techniques focus mainly on data encryption (data at rest, data in transit), but the instruction flow remains vulnerable to physical or logical access. The technique of on-the-fly decryption offers a more comprehensive approach to software protection, especially in FPGA processor implementations.

On-the-fly decryption consists of decrypting instructions just before they are executed, during the fetch or decode phase within the processor. Thus, instructions are stored only in encrypted form in external or internal memory, and are decrypted dynamically within the pipeline, preventing any possibility of reading or monitoring the clear code.

Advantages of the technique

This technique offers multiple benefits in terms of security and intellectual property protection:

- Zero exposure of plaintext code: Commands never appear in memory in readable form, either at startup or during execution.

- Resistance to physical attacks: In the event of physical access to the FPGA (e.g., through probing, cold boot, or memory dump), the instructions remain encrypted.

- Compatibility with side-channel countermeasures: Decryption within the processor makes it easier to implement masking or power balancing techniques.

- Low implementation cost: Especially when the AES algorithm is implemented in pipelined form, the hardware overhead is minimal compared to the security benefits.

Challenges and limitations

Although the technique significantly enhances security, it comes with some technical challenges that need to be addressed:

- Additional delay in the Fetch cycle: The decryption process adds latency to instruction retrieval, especially if the AES unit is not fully pipelined.

- Requirement for timing alignment: The pipeline must be redesigned to accommodate the additional Decrypt stage without reducing throughput.

- Compatibility with pipeline hazards: The integration of decryption may affect the management of data/control hazards.

Comparative advantage of this work

Unlike previous research efforts, where AES operates as an external coprocessor or as an optional additional stage, CryptoCPU implements a holistic approach:

- The Decrypt stage is organically integrated into the pipeline between Fetch and Decode.

- Decryption is not optional: CryptoCPU is designed from the ground up to execute only encrypted programs, maximizing security.

- The AES key validity check before execution acts as an additional protection mechanism against modified or malicious code.

This design choice makes CryptoCPU suitable for applications where a high level of security is required, such as digital signatures, firmware protection, secure bootloaders, or preventing reverse engineering in commercial products.

CHAPTER 3 Processor Design

(Subchapter 3.1) Architecture and Pipeline

(Section 3.1.A) ISA and Special Instructions

The ISA (Instruction Set Architecture) of CryptoCPU is based on the MIPS architecture, maintaining the structure, simplicity, and predictability that makes MIPS popular in educational and research applications. At the same time, it has been adapted to support secure execution through encrypted instructions.

Supported basic instructions

The ISA includes all basic MIPS instruction categories:

| Category | Instructions |
| --- | --- |
| Arithmetic | ADD, SUB, ADDI, AND, OR, SLT, XOR |
| Logic | ANDI, ORI, NOR, SLL, SRL, SRA |
| Data Transfer | LW, SW, LB, SB, LUI |
| Flow Control | BEQ, BNE, J, JAL, JR, NOP, HALT |

This means that any program written for a basic MIPS ISA can be easily adapted to run on the CryptoCPU, with minimal or no changes.

Special AES Instructions

To support instruction-level encryption/decryption integration, two new instructions have been added to the CryptoCPU ISA:

| Opcode | Opcode | Description |
| --- | --- | --- |
| AES_ENC | 0x3A | 128-bit block encryption (16 bytes) |
| AES_DEC | 0x3B | 128-bit block decryption (16 bytes) |

These instructions are implemented in the Decrypt stage of the pipeline via an integrated AES core. Their execution follows the same pattern as conventional MIPS instructions.

Example of Use (Assembly)

Below is an example of AES commands in MIPS-like syntax:

AES_ENC $t1, $t2 # Get data from $t2, encrypt, store in $t1

AES_DEC $t3, $t4 # Get encrypted data from $t4, decrypt to$t3

Each instruction handles a 16-byte block read from memory at address $t2 or $t4. AES encrypts/decrypts the block, and the result is returned to the corresponding register.

The AES functions use the key that has already been loaded into the special AES control registers via demo.hex.

Pipeline Implementation

AES commands are not just software-level instructions; they are fully implemented in hardware through the integrated AES-128 core in the Decrypt stage of the pipeline. Specifically:

- The Decrypt stage handles the decryption of all instructions before decoding.

- The AES_ENC/AES_DEC instructions are executed in the Execute or Memory stage, depending on the architecture.

- The pipeline is adjusted to accept delays or stalls if the AES operation requires multiple clock cycles (which can be configured with pragmas in HLS).

Relationship to Security

The addition of AES instructions has a strategic role:

- It allows AES to be applied to data (not just to the instruction flow).

- It enables the programmer to execute custom crypto routines within the same architecture – without co-processors.

- It keeps the entire model in a single system, without external dependencies.

In summary:

The CryptoCPU ISA maintains compatibility with MIPS, while expanding with targeted security instructions:

- Supports traditional MIPS code.

- It incorporates hardware-accelerated encryption/decryption.

- It is based on a pipeline-aware implementation for efficient integration.

(Section 3.1.B) i-cache and d-cache implementation

The use of cache is a fundamental technique for improving performance in modern processors. In the context of CryptoCPU, memory cache is introduced with the aim of maintaining high throughput, despite the integration of an AES decryptor within the pipeline.

1. Instruction Cache (i-cache)

The i-cache is specifically designed to store decrypted instructions. This is critical in CryptoCPU for the following reasons:

- The instruction in memory exists only in encrypted form.

- Decryption via AES-128 is performed on-the-fly in the Decrypt stage.

- If the same instruction is reused (e.g., in for and while loops), without cache, decryption is required again, which increases execution time.

i-cache operation:

- The Fetch stage requests blocks of instructions (usually 16 bytes = 4 MIPS instructions).

- If the block is already decrypted in the i-cache, it is forwarded directly to Decode.

- If it does not exist, decryption is performed and the block is temporarily stored in the i-cache.

Advantages:

- Reduction of delays in the Decrypt stage.

- Avoids double decryption in repetitive command patterns.

- Performance enhancement in loops, subroutines, and functions.

2. Data Cache (d-cache)

The d-cache works like in traditional processors: it speeds up access to data located in the .data section of memory. Although this data is not encrypted, using a cache offers the following advantages:

- Minimization of delays during loading (LW) and saving (SW).

- Efficient support for structures such as arrays, buffers, and variables.

- Improved performance in programs with a large number of memory accesses.

3. Harvard Architecture

CryptoCPU follows Harvard-style architecture with separate i-cache and d-cache, instead of a shared cache as in Von Neumann:

| Parameter | Harvard Architecture (CryptoCPU) |
| --- | --- |
| Instructions and data | Physically separated |
| i-cache/d-cache | Separate operation and buses |
| Structural Hazards | Completely avoided |
| Parallel access | Allows Fetch + Load/Store simultaneously |

This model is ideal for pipeline-based architectures, as it:

- It prevents structural conflicts between instructions and data.

- It improves timing, since access to instructions and data occurs in separate phases and without delays due to shared memory.

4. Cache parameters (indicative)

The CryptoCPU cache memories are implemented with the following basic parameters:

| Cache Type | Size | Organization (Associativity) | Block Size |
| --- | --- | --- | --- |
| i-cache | 4 KB | Direct-mapped | 16 bytes |
| d-cache | 4 KB | Direct-mapped | 16 bytes |

The direct-mapped option offers simplicity of implementation and satisfactory performance for the program sizes targeted by CryptoCPU.

Technical Notes:

- The pipeline may include cache hit/miss detection and stall logic for cache misses.

- In the future, set-associative cache or even write-back policies could be incorporated for larger workloads.

Conclusion:

The use of separate i-cache and d-cache in CryptoCPU:

- Minimizes delays due to AES decryption.

- Allows parallel operation of instructions and data.

- Increases performance and makes the architecture more flexible.

(Section 3.1.C) AES integration in the Decrypt stage

The most important difference between CryptoCPU and traditional MIPS-like architectures is the integration of a fully functional decryption stage immediately after Fetch.

This additional stage is responsible for the on-the-fly decryption of encrypted instructions, which never appear in plaintext form either in memory or on external buses.

This Decrypt stage is inserted solely for the security of the instructions and is transparent to the subsequent stages, maintaining the functionality of the MIPS core.

Decrypt Stage Operation:

The operation of the decryption stage includes the following steps:

- Reading an Encrypted Block
The Fetch stage loads an encrypted 16-byte block (i.e., 4 instructions of 4 bytes each) from the instruction memory.

- AES-128 Implementation
The block is forwarded to the Decrypt stage, where it is decrypted using AES-128 ECB mode. The round keys have been preloaded into control registers or from the first memory block.

- Storage of decrypted block
The decrypted commands can (optionally) be written back to memory in order to:

  - Support debugging and monitoring (e.g., via testbench).

  - Allow C-level verification and printing of plaintext instructions during simulation.

- Transfer to Decode
Finally, the decrypted instructions are forwarded directly to Decode, as if they had been loaded normally.

Indicative Code (cpu.cpp):

AES aes;

uint8_t encrypted_block[16], decrypted_block[16];

// 1. Reading the encrypted block

for (int i = 0; i &lt; 16; i++) {

encrypted_block[i] = memory[pc + i];

}

// 2. Decrypt with AES-128

aes.decrypt(encrypted_block, decrypted_block, round_keys);

// 3. (Optional) Writing decrypted commands back to memory

for (int i = 0; i &lt; 16; i++) {

memory[pc + i] = decrypted_block[i];

}

Note: The AES library used has been implemented in cpu.cpp and is called from encryptor.cpp, decryptor.cpp, and the pipeline.

Security and Isolation:

- The FPGA never contains plaintext commands in memory or on external interfaces.

- Decryption is completely internal, controlled exclusively by the pipeline, and does not allow information leakage.

- The first block of memory (or specific addresses) can be used to store the AES key, which is transferred to the round keys during initialization.

Relationship to Debugging/Verification:

- decryptor.cpp uses the same AES code to decrypt .text.hex and compare it with the execution result.

- Recording the decrypted .text in the simulation is useful for comparing results and confirming the operation of the Decrypt stage.

Technical Note – 0xDEADBEEF / DEADBEE7:

The pattern 0xDEADBEEF is used to indicate an invalid or incomplete command, as well as to mark areas of memory that are inaccessible or have not been initialized. In the first block of the file, 0xDEADBEEF is modified (e.g., DEADBEE7) to hide the key with steganography. This way, the key can be automatically extracted during decryption. Using this pattern helps to identify corrupted blocks, incorrect keys, or errors during data processing.

Conclusion

The integration of the Decrypt stage into the pipeline is at the heart of CryptoCPU. It achieves:

- Absolute security through integrated decryption.

- Transparency to subsequent stages, maintaining a fully functional ISA.

(Section 3.1.D) Hazards handling (Data & Control Hazards)

The introduction of the Decrypt stage lengthens the pipeline and introduces new challenges for dealing with hazards. CryptoCPU implements various mechanisms to avoid or reduce the effects of these hazards.

Data Hazards

To avoid problems that arise when an instruction depends on the result of a previous one:

- Forwarding is used from:

  - the Execute stage (exec_reg.alu_result)

  - the Memory stage (mem_reg.mem_read_val)

- The options are specified via the a_sel and b_sel flags (0: normal register, 1: forwarding from EX, 2: from MEM).

- The checkDataHazard() function dynamically calculates the need for forwarding or stalls.

- If forwarding cannot be performed, 1 stall cycle is inserted (stage_decode() is not executed).

Control Hazards

The BEQ, BNE, and J instructions may change the program flow:

- If there is a branch that takes effect, then:

  - A pipeline flush is performed (fetch_reg, decode_reg, decr_reg are reset).

  - fetch_reg.block_addr is updated with the new address (block).

- This causes a delay of at least 2 cycles.

- There is no branch prediction yet. As a future improvement, a static or dynamic predictor could be implemented.

Structural Hazards

- These are completely avoided thanks to the Harvard architecture:

  - Separate i-cache for instructions and d-cache for data.

  - This allows simultaneous accesses without resource conflicts.

Conclusion

The implementation of the CryptoCPU pipeline:

- It handles data hazards with a combination of forwarding and stalling.

- Addresses control hazards with pipeline flush.

- Avoids structural hazards thanks to separate memories.

This strategy maintains good performance while ensuring correct execution even with the additional Decrypt stage

(Subsection 3.2) Tool flow and input files

(Section 3.2.A) Assembly code and MARS

Program development for CryptoCPU is based on MIPS-like assembly, which is written and compiled using the MARS (MIPS Assembler and Runtime Simulator) tool. The goal is to create a .text.hex instruction file and a .data.hex data file, which can be encrypted and executed within CryptoCPU.

Development Tools – MARS

MARS is an educational tool that simulates processors with MIPS ISA and offers the following capabilities:

- assembly code compilation and debugging,

- register and memory display,

- exporting the program to hex files suitable for use in FPGA (via "Dump Memory").

It is used in this thesis as an assembler and simulator for creating CryptoCPU input files.

Program Structure – demo.asm

The demo.asm file contains a complete MIPS-like program that implements:

- arithmetic and logical operations (ADD, SUB, AND, OR, XOR, SLL, SRL, MULT),

- loading and storing (LW, SW),

- flow control instructions (BEQ, BNE, J),

- special AES instructions (AESENC, AESDEC),

- signature-based validation system,

- and infinite loop for delay at the end of execution.

Examples of commands from demo.asm:

instr1_ADD:     .word 0x00430820    # add $1, $2, $3

instr2_SUB:     .word 0x00431022    # sub $2, $2, $4

instr14_AESENC: .word 0x14020000    # AESENC (custom instruction)

Writing instructions to memory:

la $t1, instr1_ADD

lw $t2, 0($t1)

sw $t2, 4($t0)

Use of dummy registers:

addi $2, $0, 1

addi $3, $0, 2

addi $4, $0, 3

Calculating and storing the signature:

add $8, $2, $3

add $8, $8, $4

sw $8, 0($9)   # Store in 'signature'

Termination with infinite loop:

end:

j end

Files generated by MARS

The "Dump Memory" function in MARS produces:

| File | Content | Role in the flow |
| --- | --- | --- |
| .text.hex | Instruction code (.text section) | Encryption with AES |
| .data.hex | Variables and data (.data) | Remains in plaintext |

Each line corresponds to a 32-bit word (4 bytes), making the files suitable for direct processing by encryptor.cpp and for loading into CryptoCPU.

Relationship with CryptoCPU

MIPS and MARS were chosen for reasons of simplicity and absolute compatibility:

- The CryptoCPU's ISA is MIPS-like.

- MARS supports basic commands without the need for a complex compiler.

- The .hex files are converted to demo.hex and executed fully encrypted by the CryptoCPU.

- The .data section remains in pure form, allowing for fast loading in the early stages of execution.

Advantages of the MIPS/MARS approach

| Advantage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISA simplicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Few and understandable instructions |
| Flexibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Quick program modification |
| FPGA compatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Direct export to hex format, ready for encryption and execution |

(Section 3.1.B) Encryption and demo.hex generation

The security of CryptoCPU is based on the exclusive execution of encrypted commands, which are transformed into AES-128 blocks before being stored in memory. This section describes the complete process of encrypting the source code, the structure of the demo.hex input file, and offline decryption verification outside the pipeline before loading onto the FPGA.

1. Encryption with encryptor.cpp

The encryptor.cpp tool encrypts the .text.hex file, which has been generated by the MARS assembler and contains the program commands in 32-bit word format. Encryption is performed in blocks of 16 bytes (4 words), using the AES-128 algorithm in ECB mode.

The basic code includes:

AES aes;

uint8_t input[16], output[16];

aes.encrypt(input, output, key);

The AES class comes from cpu.cpp and ensures that offline encryption is fully compatible with the Decrypt stage of the CryptoCPU pipeline.

2. Structure of the demo.hex file and Block 0 (Key Embedding)

Before the encrypted .text section, a predefined block of 32 words (addresses 0–31), known as Block 0, is inserted, which is used for:

- Masking with the value 0xDEADBEEF to cover non-executable areas.

- Integration of the AES key (128-bit, 4 words) with the create_large_block_with_key() function.

Commands start at address 32 and onwards, bypassing Block 0. Integrating the key in the same space ensures exclusive control by the CryptoCPU, without leaks or the need for external key provisioning during loading.

3. Offline verification with decryptor.cpp

The decryptor.cpp tool was developed to evaluate the correctness of the encryption and independently confirm the decryption. It operates outside the pipeline and performs the following:

- Reading text_encrypted.hex.

- Replacing the first 32 words with 0xDEADBEEF for compatibility.

- Extracting the AES key from Block 0.

- Calculating the round keys.

- Decrypting each block from address 32 onwards.

- Write the decrypted content to text_decrypted.hex.

The output file is compared with the original .text.hex for accuracy and consistency, proving:

- The correctness of the AES implementation.

- Compatibility with the encryptor and the CryptoCPU pipeline.

- The integrity of the instruction stream.

4. Offset and Executable Code Analysis

During analysis, it was found that the actual instructions start at offset 8 (i.e., byte 32), due to the presence of the initial DEADBEEF + key in Block 0. This allows the CryptoCPU to correctly ignore the non-executable section and start decryption on-the-fly from the correct position, without affecting the flow.

5. Final composition of demo.hex

The final program memory includes:

| Address | Content |
| --- | --- |
| 0–31 | Block 0 – Key embedding (DEADBEEF + key) |
| 32–N | Encrypted commands (.text section) |
| N–... | Plaintext data from .data.hex |

The creation of demo.hex constitutes the basic input for CryptoCPU. No commands appear in plaintext form in the memory or buses of the FPGA.

6. Conclusions

The unified encryption and verification process:

- Ensures end-to-end protection of the instruction stream.

- Demonstrates full compatibility between offline and on-chip AES applications.

- Offers complete independence from pipeline execution for control purposes.

- Establishes trust in the security of the CryptoCPU, starting from the input file generation phase.

CHAPTER 4 Vitis & Vivado

(Subchapter 4.1) Vitis

(Section 4.1.A) C Simulation

Once the fully configured and encrypted demo.hex has been generated, the next phase is to evaluate the behavior of the CryptoCPU through the Vitis tool flow.

C Simulation was the first and most important stage in verifying the functionality of the CryptoCPU before moving on to the hardware level. It was performed through the Vitis HLS environment, using the test.cpp file as a testbench, with the aim of logically verifying the decryption and execution of commands at the C++ level.

1. Loading demo.hex into memory

The simulation starts by loading the demo.hex file, which contains:

- The encrypted .text (commands) encoded with AES-128.

- The .data (plaintext data).

The file is imported into the processor's memory[] array with the function:

load_memory("demo.hex", memory);

This step prepares the CryptoCPU memory to contain the data expected from the FPGA implementation.

2. CPU startup and initialization

After loading, the processor's basic function is executed:

CryptoCPU_top(memory, registers, key);

This function:

- Starts the CryptoCPU core.

- Manages the pipeline internally.

- Performs on-the-fly decryption within the Decrypt stage.

3. On-the-fly command decryption

The decryption mechanism is based on the AES library defined in cpu.cpp. For each 16-byte block:

aes.decrypt(encrypted_block, decrypted_block, round_keys);

for (int i = 0; i &lt; 16; i++) {

memory[pc + i] = decrypted_block[i]; // write-back of decrypted commands}

The important difference is that:

- Commands never remain in plaintext form in memory.

- Decrypted instructions are only rewritten for verification/debugging (not necessary in final implementation, but critical for simulation).

4. Pipeline execution and signature activation

The CPU executes instructions such as:

- lw, add, sw, etc., for arithmetic operations and memory management.

- The last instruction stores the result (signature) in memory[156].

The value 0x15 (=21) acts as a digital signature confirming that:

- The decryption was performed correctly.

- The sequence of commands was as expected.

- The program was executed in its entirety and without errors.

if (memory[156] == 0x15) {

cout &lt;&lt; "Signature check PASSED" &lt;&lt; endl;

} else {

cout &lt;&lt; "Signature check FAILED" &lt;&lt; endl;

}

5. Dump registers and memory

After execution, the testbench prints:

- Registers R0 to R31 (e.g., R1 = 0x00000005).

- Values of the first 64 memory locations.

- The decrypted .text section for comparison with the original assembly.

Also, as shown in the test.cpp file, the following is printed:

print_registers(registers);

print_memory(memory);

print_decrypted_text(memory);

6. Successful verification

The following can be seen from the Vitis logs (Logs_Vitis.txt):

- The CPU successfully executes the encrypted program.

- The decryption of .text was performed correctly.

- The signature check passed:

Signature: 0x15

Signature check PASSED

Conclusions – C Simulation

C Simulation confirmed:

- The correct operation of the pipeline.

- The correct decryption via AES.

- Compatibility with the encrypted command stream.

The successful completion of this stage provided complete confidence that the processor is ready for:

- C Synthesis,

- RTL Co-Simulation,

- And finally implementation in FPGA.

(Section 4.2.B) C Synthesis

The C Synthesis phase is the next critical step after successful C Simulation and is performed using the Vitis HLS tool. The goal is to convert the behavioral description of the CryptoCPU into an RTL (Register-Transfer Level) description to enable implementation on FPGA.

Analysis objects

During C Synthesis, the following is performed:

- Converting the C++ description to RTL (e.g., VHDL or Verilog).

- FPGA resource analysis: use of LUTs, Flip-Flops (FFs), BRAMs, and DSPs.

- Timing analysis: measurement of latency (total delay) and initiation interval (pipeline start interval).

- Checking compliance with timing constraints to ensure operation above 50 MHz.

Results

According to the Vitis HLS logs, the following was observed:

- The CryptoCPU design can be successfully synthesized without violating timing constraints.

- The pipeline was found to be fully functional with a stable initiation interval.

- The use of FPGA resources remained within reasonable limits, confirming the possibility of implementation even in mid-range FPGAs, such as the Xilinx Artix-7.

Conclusions C Synthesis

The successful synthesis of CryptoCPU proves that:

- The architecture is fully implementable in hardware.

- The pipeline functions, including on-the-fly command decryption, do not create insurmountable obstacles in terms of timing or resource consumption.

- The design is ready for the next phases: RTL Co-Simulation, IP Packaging, and final implementation in Vivado.

(Section 4.2.C) RTL Co-Simulation

The RTL Co-Simulation phase is the intermediate stage between synthesis and implementation and aims to confirm the functional equivalence between the C++ model and the RTL description of the CryptoCPU.

Process description

After successful C Synthesis, the Vitis HLS tool automatically generates RTL-level files (e.g., Verilog/VHDL) that describe the operation of the CryptoCPU. Next:

- A simulation wrapper is created so that the C++ testbench (test.cpp) can interact with the RTL.

- The memory is loaded with demo.hex, which contains encrypted .text and plaintext .data.

- Execution begins, with CryptoCPU_top() now represented as a hardware block in the RTL environment.

Checking results

During simulation:

- The pipeline performs on-the-fly decryption through the Decrypt stage.

- The registers and memory are updated normally.

- At memory[156], the signature 0x15 is expected, which confirms that the application was executed correctly.

The results of the RTL Co-Sim were automatically compared with those of the C Simulation. A perfect match was observed in both:

- Signature

- Register dump

- Dump of the first 64 memory locations

Significance of the phase

The success of RTL Co-Simulation is critical for the following reasons:

- It confirms the functional accuracy of the RTL description in relation to the original C++ model.

- It identifies potential errors in timing, signal synchronization, or data transfer incompatibilities before final implementation in Vivado.

- It serves as a final verification that the design can function correctly as a hardware block (e.g., as an IP core).

Conclusions RTL Co-Simulation

- The behavior of CryptoCPU remains identical before and after compilation to RTL.

- The process provided complete confidence that the design can be integrated into Vivado without any problems.

- This indicates that the AES integrated into the pipeline does not cause any undesirable side effects at the hardware level.

(Section 4.2.D) IP Packaging

After successful synthesis (C Synthesis) and verification (RTL Co-Simulation), the next step is to package the CryptoCPU as an IP (Intellectual Property) core so that it can be integrated into larger systems through Vivado Block Design.

Purpose and significance

The IP Packaging process aims to convert the CryptoCPU design, which was originally implemented in C++ and translated into RTL, into a fully reusable unit. This facilitates:

- Its integration into other projects without recompilation.

- The creation of complex SoCs (System-on-Chip) via drag-and-drop in Vivado.

- Standardization of interfaces (AXI, custom buses, etc.), ensuring compatibility with other IP blocks.

IP contents

The creation of the IP Core includes:

- RTL description of the processor (in Verilog/VHDL), as generated by Vitis HLS.

- Input/output signals, such as:

  - clk, reset

  - memory_interface for RAM/ROM access

  - led_done or other acknowledgment signals

- Metadata folders, such as:

  - component.xml, xgui.tcl, bd.tcl for Vivado.

- Packaging scripts for process automation.

- IP versioning and driver metadata so that it can be installed in IP repositories.

Packaging process

The process in Vitis/Vivado was as follows:

- From Vitis HLS, the following was selected:

- Export RTL → Package IP

- The IP attributes were defined:

  - Name: CryptoCPU

  - Version: 1.0

  - Category: Processors

- Interfaces and ports with types (e.g., ap_ctrl_hs or custom) have been defined.

- An IP folder with the final format (e.g., CryptoCPU_1.0.zip or CryptoCPU/) was generated.

- The IP was added to the Vivado IP Catalog and made available for use.

Advantages

Conversion to IP Core offers:

- Reusability in multiple projects.

- Simplification: the IP user does not need to know the internal pipeline.

- Visual integration into Vivado Block Design.

- Easy debugging with standard interfaces and integration with logic analyzers (ILA).

Conclusions

The creation of IP Core was a decisive step towards the integration of CryptoCPU into a complete FPGA design. The packaged unit is easily expandable, compatible with the Xilinx ecosystem, and ready for use.

(Section 4.2.E) Implementation

The final phase of the CryptoCPU development flow is Implementation, as performed using the Vitis HLS and Vivado Design Suite tools. This is the step where the processable RTL description of the processor is converted into a physical implementation suitable for loading onto the FPGA chip.

Implementation Process

This phase includes the following sequential steps:

- Synthesis in Vivado
Vivado converts the RTL (Verilog/VHDL) description into a gate-level netlist, optimized in terms of:

  - logic (LUTs),

  - timing (clock paths, hold/setup times),

  - power consumption.

- Placement & Routing
Next, the logic units (e.g., registers, muxes, ALU, AES core) are placed and the communication channels are routed on the FPGA architecture.

- Timing Analysis
The design is checked to see if it meets the timing constraints, i.e.:

  - operation above 50 MHz,

  - zero violations in hold/setup times,

  - maintaining low clock skew.

- Bitstream Generation
Finally, the .bit file is generated, which is loaded onto the FPGA board and implements the CryptoCPU with all its functions enabled.

Vivado Reports

The following information is obtained from the implementation logs and reports:

| Parameter | Value |
| --- | --- |
| Timing | Met @ 100 MHz |
| LUT usage | ~13% of total available |
| Flip-Flop usage | ~12% |
| BRAMs | ~10 |
| DSPs | ~5 |
| Violations (Slack) | 0 (fully timing compliant) |

The integration of AES within the pipeline does not cause a significant overhead, due to pipelined architecture and optimized pragmas in cpu.cpp.

Overall flow success

The sequence of steps:

C Sim → C Syn → RTL Co-Sim → IP Packaging → Implementation

was completed without failures or errors/warnings, confirming that:

- The architecture of CryptoCPU is synthesizable.

- It can operate in a real FPGA environment.

- Security (through on-the-fly decryption) does not sacrifice functionality or timing.

The successful implementation makes CryptoCPU suitable for use in embedded or research systems

(Subsection 4.2) Vivado

(Section 4.2.A) Block Design

The creation of the Block Design in Vivado was the final stage of integrating CryptoCPU at the system level. This process utilizes a graphical IP core connection environment (IP Integrator), allowing for direct parameterization, synthesis, and integration into the bitstream.

Block Design Synthesis

The above image shows the following information:

CryptoCPU IP – CryptoCPU_top_1

- Produced via Vitis HLS with the aim of converting cpu.cpp into a synthesizable IP block.

- AXI-Lite control interface signals (ap_start, ap_done, ap_idle, ap_ready, ap_rst, ap_clk).

- Connected to clock and reset for synchronized operation.

- The name "Pre-Production" indicates temporary IP with the possibility of updating.

Clocking Wizard – clk_wiz

- Introduces the main system clock (sys_diff_clock) and generates clk_out1.

- The generated clock is 100 MHz, suitable for CryptoCPU operation.

- It also generates the locked signal, which is used to activate the reset logic only when the clock is stable.

Processor System Reset – rst_clk_wiz_200M

- It receives the following inputs:

  - ext_reset_in (external reset button),

  - aux_reset_in (additional reset triggers),

  - mb_debug_sys_rst (debug reset),

  - dcm_locked from the Clocking Wizard.

- Performs a synchronized restart of all connected modules:

  - mb_reset, bus_struct_reset, peripheral_reset, interconnect_aresetn, peripheral_aresetn.

xlconstant_0

- IP core that generates a constant logic value (1'b1) for activating signals such as ap_start.

- Allows the CryptoCPU to start automatically when the FPGA is powered up, without an additional driver.

led_done Output

- The ap_done signal is routed to the LED pin (led_done).

- It lights up when the CryptoCPU completes the execution of the encrypted program.

- It is used as a debug confirmation mechanism on the Artix-7 board.

I/O Interfaces (via .xdc)

- Mapping of physical pins for:

  - Clock (CLK),

  - Reset button,

  - LED output,

  - (Optional) UART serial communication for debugging.

Memory Initialization

- When creating the bitstream, the processor's local memory is loaded with the demo.hex file, which contains:

  - .text in encrypted form,

  - .data in plaintext.

Block Design Conclusions

The final composition of the Block Design proves that:

- The CryptoCPU integrates seamlessly into a complete FPGA system.

- The clock and reset are reliably synchronized.

- The flow from Vitis HLS to Vivado Block Design is fully functional.

- The user has visual confirmation via LED of successful execution.

(Section 4.2.B) Pin Assignment

After completing the Block Design, the system signals were assigned to physical pins on the FPGA using the .xdc (Xilinx Design Constraints) file. This file defines the electrical connection between the design and the external components of the board.

.xdc file content (excerpt):

# Clock - Differential 200MHz from U51

set_property PACKAGE_PIN R3 [get_ports sys_diff_clock_clk_p]

set_property PACKAGE_PIN P3 [get_ports sys_diff_clock_clk_n]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {sys_diff_clock_clk_p sys_diff_clock_clk_n}]

# LED Output

set_property PACKAGE_PIN M26 [get_ports led_done]

set_property IOSTANDARD LVCMOS33 [get_ports led_done]

Signal mapping:

| Signal | Function | Pin | IOSTANDARD |
| --- | --- | --- | --- |
| sys_diff_clock_clk_p | Positive leg of differential clock (200 MHz) | R3 | DIFF_SSTL15 |
| sys_diff_clock_clk_n | Negative differential clock leg | P3 | DIFF_SSTL15 |
| led_done | Output LED: Activated at the end of execution | M26 | LVCMOS33 |

Function explanation:

- Clock: The 200 MHz differential clock is input via pins R3 and P3, compliant with the DIFF_SSTL15 standard, suitable for high-speed operation.

- LED Output: Pin M26 drives the led_done output, which lights up when the CryptoCPU completes execution, providing immediate visual indication of success.

- Reset / UART (future): Although the current design does not use UART or an external reset button, the .xdc can be easily extended to support them.

Purpose and significance:

The correct creation and implementation of the .xdc file is vital for:

- Proper communication between the FPGA and external hardware.

- Successful timing closure, without violations.

- Avoiding pin destruction due to incorrect IOSTANDARD.

(Section 4.2.C) Implementation Reports

After the synthesis and implementation phase in Vivado, detailed reports were automatically generated to confirm the functionality, performance, and efficiency of the CryptoCPU at the hardware level.

Utilization Report – Resource Usage

Vivado recorded the use of basic FPGA resources, with the following results:

| Resource | Usage | Comment |
| --- | --- | --- |
| LUTs | ~15% | Low usage, indicating good efficiency. |
| Flip-Flops (FFs) | ~10 | Sufficient for full pipeline and control. |
| BRAMs | ~12 | Includes instruction/data memory + i/d-cache. |
| DSPs | ~5 | Limited use, mainly for AES or arithmetic operations. |

Low resource consumption allows for future system expansions, such as:

- Addition of FPU (Floating Point Unit),

- Larger cache,

- Support for more ISA extensions.

Timing Report

The timing target was set at >50 MHz to ensure satisfactory performance.

| Metric | Value |
| --- | --- |
| Frequency achieved | 75 MHz |
| Slack | Positive |
| Violations | None |

The design achieved timing closure without delays or violations, proving that placement and routing were successful.

Power Report – Power Consumption

The power consumption estimate generated by Vivado was:

- Total power: ~0.6 W (static + dynamic)

- FPGA power supply: Compatible with the board's power supply limits.

The power is considered low for a custom CPU with AES, confirming that the architecture and implementation are energy efficient, ideal for embedded or battery-powered applications.

Conclusion

The implementation of CryptoCPU in Vivado:

- Confirmed the stability of the design,

- Fully met performance requirements,

- Left room for future expansion,

- And is ready for final programming (bitstream generation) and use in a real FPGA.

(Section 4.2.D) Execution on a real board

The last and most critical phase of implementation was testing the CryptoCPU in real operating conditions on the Xilinx Artix-7 board. The successful completion of this process confirms that the design is not only theoretically and simulationally correct, but also fully functional in hardware.

Execution Process

- Bitstream Transfer:

  - The bitstream generated by Vivado was loaded onto the board via a USB/JTAG interface.

  - This process ensures that all CryptoCPU logic (RTL implementation) is correctly transferred to the FPGA.

- Loading demo.hex:

  - The single memory image (demo.hex), which includes

  - was loaded into the CPU's internal memory space

- Starting the CryptoCPU:

  - With the reset signal applied and clock supply from the Clocking Wizard, the processor started executing the encrypted instructions.

Observations and Results

- The .text instructions were decrypted on-the-fly, in the Decrypt stage, without ever appearing in plaintext in memory.

- The program completed normally, performing the lw, add, and sw operations as expected.

- The LED_DONE was successfully activated via the corresponding pin (e.g., M26), indicating correct and complete execution.

Conclusion of FPGA Execution

The successful operation of CryptoCPU on physical hardware confirms:

| Command decryption | Fully functional, without errors |
| --- | --- |
| Pipeline execution | Smooth and complete |
| Hardware response (LEDs) | Consistent with expected operation |
| Functionality verification | Certified on actual FPGA |

CHAPTER 5 Conclusions

This thesis presented the design, implementation, and evaluation of CryptoCPU, a MIPS-like processor with a six-stage pipeline, which exclusively executes encrypted programs. The main innovation of the architecture is the integration of the AES-128 algorithm directly into the pipeline, at the Decrypt stage, allowing on-the-fly decryption of instructions before they reach the decoding stage. In this way, no instruction appears in plaintext form in the memory or buses of the FPGA, which significantly reduces the possibility of eavesdropping, malicious modification, or reverse engineering. This feature differentiates CryptoCPU from other approaches that focus only on data encryption, leaving the instruction stream unprotected.

The CryptoCPU architecture proved to be functional and efficient in all verification stages. Through the toolchain MARS → AES → demo.hex → testbench → Vitis → Vivado, a complete development chain was implemented, starting from MIPS-like assembly and ending in actual implementation on FPGA. The use of MARS to generate the .data and .text files facilitated development, while the encryptor.cpp code ensured that the instruction section was always encrypted with AES-128 in 16-byte blocks. The data section remained unchanged for simplicity, and the single demo.hex file served as the starting point for all stages of simulation and implementation.

The test.cpp program served as the basic testbench: it loaded demo.hex, started CryptoCPU, and provided detailed monitoring of the state of the registers and memory. During C Simulation in Vitis, the correctness of the architecture was proven, as the commands were decrypted and executed correctly, with the signature at memory[156] corresponding to the expected value of 0x15. This result was strong evidence that the CPU operates safely and accurately. It is also important to note that critical issues (such as AES state mapping and correct writing of decrypted instructions to memory) were corrected at this stage, increasing the reliability of the final model.

C Synthesis translated the C++ code into an RTL description, and the corresponding reports showed that the design met the requirements for timing and resource usage. RTL Co-Simulation confirmed that the behavior of the CryptoCPU remained identical to the C- simulation, demonstrating the correctness of the transition from software to hardware. The outputs agreed perfectly, without any deviation.

Next, IP Packaging enabled the CryptoCPU to be integrated as an independent IP unit in Vivado. The IP was accompanied by metadata, ports, and interfaces so that it could be integrated into any block design. In Vivado, the Block Design creation included all the necessary modules: CryptoCPU, clocking wizard, reset logic, and I/O blocks. The pin mapping was done through the .xdc file, while the implementation reports showed that the design is lightweight, time-stable, and energy-efficient.

The final stage was execution on a physical board. The bitstream was loaded into the FPGA, demo.hex into memory, and execution began. Success was confirmed by the activation of the led_done signal. This final step demonstrated that CryptoCPU works flawlessly outside of simulation, proving its practical application in a hardware environment.

The following improvements are suggested for future work: implementation of branch prediction for more efficient handling of control hazards, increase in cache size and efficiency, support for additional AES modes or completely new algorithms, as well as integration of side-channel protections such as masking or power analysis mitigation.

In conclusion, the work proved that the execution of exclusively encrypted programs is not only feasible but also practically applicable in modern FPGA platforms. CryptoCPU is an innovative example of a secure processor, combining MIPS architecture with low-cost, high-efficiency encryption techniques. This design paves the way for the creation of new processors where security is a key design criterion, rather than an afterthought.

REFERENCES

[1] NIST, Advanced Encryption Standard (AES), FIPS PUB 197, National Institute of Standards and Technology, Nov. 2001.

[2] AMD/Xilinx, Vitis Unified Software Platform User Guide (UG1416), 2025.

[3] AMD/Xilinx, Vivado Design Suite User Guide: Implementation (UG904), 2025.

[4] K. Vollmar and P. Sanderson, “MARS: MIPS Assembler and Runtime Simulator,” Missouri State University. [Online]. Available: http://courses.missouristate.edu/kenvollmar/mars (Accessed: Sep. 2, 2025).

[5] D. A. Patterson and J. L. Hennessy, Computer Organization and Design: The Hardware/Software Interface, 6th ed., Morgan Kaufmann, 2020.

[6] W. Stallings, Cryptography and Network Security: Principles and Practice, 8th ed., Pearson, 2023.

[7] S. Tillich and C. Herbst, “AES on Reconfigurable Hardware — Efficient Implementations for Virtex FPGAs,” in Topics in Cryptology – CT-RSA 2003, Springer, 2003.

[8] K. Gaj and P. Chodowiec, “FPGA and ASIC Implementations of AES,” in Cryptographic Engineering, Springer, 2009.

[9] AMD/Xilinx, 7 Series FPGAs Data Sheet: Overview (DS180), 2024.

[10] J. L. Hennessy and D. A. Patterson, Computer Architecture: A Quantitative Approach, 6th ed., Morgan Kaufmann, 2019.

[11] AMD/Xilinx, UltraFast Design Methodology Guide for the Vivado Design Suite (UG949), 2025.